Dial plans and routing

Dial plans and routing One page pulled from the Canon rooms named in the source column. Open the room before you quote a number in a design. This is not a copy of a Cisco document. Check What the room says Source What is the default extension length? 4 digits, and the allowed range is 2–10. [cite:1] T2-R03-dial-plans-pstn-routing.html What three PSTN choices can a location have? Cisco PSTN, Cloud Connected PSTN, or premises-based PSTN. [cite:2] T2-R03-dial-plans-pstn-routing.html Who can place a call through cloud PSTN? Webex Calling users. Premises-originated calls cannot. [cite:2] T2-R03-dial-plans-pstn-routing.html What is an ESN? Location routing prefix plus extension, used when inter-location extension dialing is disabled. [cite:1] T2-R03-dial-plans-pstn-routing.html How many trunks can a route group hold? 1 to 10. [cite:2] T2-R03-dial-plans-pstn-routing.html What does priority level 1 on two trunks mean? Calls are distributed randomly across them. Trunk 3 at level 2 is the fallback when a level-1 trunk is unavailable. [cite:2] T2-R03-dial-plans-pstn-routing.html Where may ! appear? Once, at the end, and only on an E.164 pattern. [cite:2] T2-R03-dial-plans-pstn-routing.html Back to the lab · Cheat Codes

Sources and go deeper

Wikipedia: Webex (Wikipedia) — https://en.wikipedia.org/wiki/Webex — last checked 2026-10

Vendor doc: Webex Help Center — https://help.webex.com/ — last checked 2026-10

DevDocs: Webex developer documentation — https://developer.webex.com/ — last checked 2026-10

DevDocs: Webex developer getting started — https://developer.webex.com/docs/getting-started — last checked 2026-10

Community: Webex community — https://help.webex.com/community — last checked 2026-10

SRND: Cisco Collaboration design zone — https://www.cisco.com/c/en/us/solutions/enterprise/design-zone-collaboration/index.html — last checked 2026-10

RFC: RFC 3261 SIP: Session Initiation Protocol — https://www.rfc-editor.org/rfc/rfc3261 — last checked 2026-10

Port table: IANA service name and port number registry — https://www.iana.org/assignments/service-names-port-numbers/service-names-port-numbers.xhtml — last checked 2026-10

Port table: Port reference for Webex Calling — https://help.webex.com/article/b2exve — last checked 2026-10

RFC: RFC 3263 Locating SIP Servers — https://www.rfc-editor.org/rfc/rfc3263 — last checked 2026-10

RFC: RFC 3264 An Offer/Answer Model with SDP — https://www.rfc-editor.org/rfc/rfc3264 — last checked 2026-10

Wikipedia: Session Initiation Protocol (Wikipedia) — https://en.wikipedia.org/wiki/Session_Initiation_Protocol — last checked 2026-10

RFC: RFC 3550 RTP — https://www.rfc-editor.org/rfc/rfc3550 — last checked 2026-10

RFC: RFC 3711 SRTP — https://www.rfc-editor.org/rfc/rfc3711 — last checked 2026-10

RFC: RFC 4566 SDP — https://www.rfc-editor.org/rfc/rfc4566 — last checked 2026-10

RFC: RFC 4733 RTP payload for DTMF — https://www.rfc-editor.org/rfc/rfc4733 — last checked 2026-10

Wikipedia: Real-time Transport Protocol (Wikipedia) — https://en.wikipedia.org/wiki/Real-time_Transport_Protocol — last checked 2026-10

Release notes: What's new in Webex Calling — https://help.webex.com/article/278bz2 — last checked 2026-10

Vendor doc: help.webex.com article njvdjf2 — https://help.webex.com/article/njvdjf2 — last checked 2026-10

Vendor doc: help.webex.com article n0xb944 — https://help.webex.com/article/n0xb944 — last checked 2026-10

Independent, free, learning-only student project. Not affiliated with or endorsed by any vendor.

Trademarks belong to their owners.

Concerns: https://cyphermonkeylabs.com/#contact

CypherMonkey Labs

© 2026 CypherMonkeyLabs

https://cyphermonkeylabs.com

https://github.com/CypherMonkeyLabs

https://linktr.ee/cyphermonkey