Survivability gateway

Survivability gateway One page pulled from the Canon rooms named in the source column. Open the room before you quote a number in a design. This is not a copy of a Cisco document. Check What the room says Source Where do endpoints register in Active mode? To the Webex cloud. [cite:1] T2-R07-survivability-gateway.html Where do they register in Survivability mode? To the local Survivability Gateway. [cite:1] T2-R07-survivability-gateway.html How long must the cloud path be back? At least 30 seconds. [cite:1] T2-R07-survivability-gateway.html What IOS starts the feature set? IOS XE Dublin 17.12.3. Hunt group, call forward, and auto attendant start at 17.18.2. [cite:1] T2-R07-survivability-gateway.html What certificate types are refused? Private or enterprise CAs, and wildcard certificates. [cite:1] T2-R07-survivability-gateway.html What hunt-group cap applies in this mode? 100 groups, 32 users each. Weighted routing is not supported. [cite:1] T2-R07-survivability-gateway.html What does Webex App support during the outage? Internal extensions, PSTN in and out, outgoing emergency, hold and resume, transfer, inbound caller ID, and shared call appearance. [cite:2] T2-R07-survivability-gateway.html Back to the lab · Cheat Codes

Sources and go deeper

Wikipedia: Webex (Wikipedia) — https://en.wikipedia.org/wiki/Webex — last checked 2026-10

Vendor doc: Webex Help Center — https://help.webex.com/ — last checked 2026-10

DevDocs: Webex developer documentation — https://developer.webex.com/ — last checked 2026-10

DevDocs: Webex developer getting started — https://developer.webex.com/docs/getting-started — last checked 2026-10

Community: Webex community — https://help.webex.com/community — last checked 2026-10

SRND: Cisco Collaboration design zone — https://www.cisco.com/c/en/us/solutions/enterprise/design-zone-collaboration/index.html — last checked 2026-10

RFC: RFC 3261 SIP: Session Initiation Protocol — https://www.rfc-editor.org/rfc/rfc3261 — last checked 2026-10

Port table: IANA service name and port number registry — https://www.iana.org/assignments/service-names-port-numbers/service-names-port-numbers.xhtml — last checked 2026-10

Port table: Port reference for Webex Calling — https://help.webex.com/article/b2exve — last checked 2026-10

RFC: RFC 3263 Locating SIP Servers — https://www.rfc-editor.org/rfc/rfc3263 — last checked 2026-10

RFC: RFC 3264 An Offer/Answer Model with SDP — https://www.rfc-editor.org/rfc/rfc3264 — last checked 2026-10

Wikipedia: Session Initiation Protocol (Wikipedia) — https://en.wikipedia.org/wiki/Session_Initiation_Protocol — last checked 2026-10

RFC: RFC 3550 RTP — https://www.rfc-editor.org/rfc/rfc3550 — last checked 2026-10

RFC: RFC 3711 SRTP — https://www.rfc-editor.org/rfc/rfc3711 — last checked 2026-10

RFC: RFC 4566 SDP — https://www.rfc-editor.org/rfc/rfc4566 — last checked 2026-10

RFC: RFC 4733 RTP payload for DTMF — https://www.rfc-editor.org/rfc/rfc4733 — last checked 2026-10

Wikipedia: Real-time Transport Protocol (Wikipedia) — https://en.wikipedia.org/wiki/Real-time_Transport_Protocol — last checked 2026-10

Release notes: What's new in Webex Calling — https://help.webex.com/article/278bz2 — last checked 2026-10

Vendor doc: help.webex.com article d68vi1 — https://help.webex.com/article/d68vi1 — last checked 2026-10

Vendor doc: help.webex.com article dgjdi9 — https://help.webex.com/article/dgjdi9 — last checked 2026-10

Vendor doc: help.webex.com article w0qmeu — https://help.webex.com/article/w0qmeu — last checked 2026-10

Independent, free, learning-only student project. Not affiliated with or endorsed by any vendor.

Trademarks belong to their owners.

Concerns: https://cyphermonkeylabs.com/#contact

CypherMonkey Labs

© 2026 CypherMonkeyLabs

https://cyphermonkeylabs.com

https://github.com/CypherMonkeyLabs

https://linktr.ee/cyphermonkey